A very minimal guide o use GRAPE-4

J. Makino Nov 16 1998

You need to use the following three functions:

--- attach GRAPE-4 hardware to the current process

call h3open

h3open_(); (in C)

---  calculate force

call accel_by_harp3_separate_noopen(ni, xi,

$     nj,xj,mj,a,p,eps2)

accel_by_harp3_separate_noopen_(ni,xi, nj,xj,mj,a,p,eps2);

(in C)

integer NI

int * ni;

: number of i-particles (which receive the force)

THIS COULD BE AN ARBITRARY NUMBER, not limited by the

physical number of chips on the board. This routine will

take care of the differ3ence

real*8 XI(3,NI)

double xi[][3];

: locations of i-particles

integer NJ : number of j-particles (which exert the force)

This should not exceed (128k/3*NBOARDS), the phyisical

memory size of the GRAPE-4 system (if you use treecode,

it's pretty unlikely that you have more than 42k

j-particles...

real*8 XJ(3,NJ): locations of j-particles

real*8 MJ(NJ)  : mass of j-particles

real*8 A(3,NJ): calculated accelerations

real*8 P(NJ)  : calculated potential energies (includes

self-interaction)

real*8 eps2   : softening length squared

NOTE:  when you use this function, MAKE THE SIZE OF the arrays for XI,

A and P LARGER THAN what is actually used, by at least 96 (the

physical number of pipelines). This routine might write some garbage

in locations after NI.

--- release GRAPE-4

call h3close

h3close_();

(in C)

sample file minimal.f :

program demo_2body

integer ndim, nmax, ni, nj

parameter(ndim=3,nmax=1024)

real*8 x(ndim,nmax),a(ndim,nmax),p(nmax),m(nmax),eps2

integer i,k

ni = 2

nj = 2

do i = 1,ni

do k = 1, ndim

x(k,i) = 0

end do

m(i) =1

end do

x(1,1) = 1

x(1,2) = -1

eps2 = 0.01

call h3open

call accel_by_harp3_separate_noopen(ni, x, nj,x,m,a,p,eps2)

do i = 1, ni

write(6,600)'X ',(x(k,i),k=1,3), ' M ', m(i)

write(6,600)'A ',(a(k,i),k=1,3), ' P ', p(i)

600     format(A, 3g17.8, A, g17.8)

end do

call h3close

end

sample file minimal.c

/*

* minimal.c

* minimal source to demonstrate the use of GRAPE-4

*/

#define NDIM 3

#define NMAX 1024

double  x[NMAX][NDIM];

double  a[NMAX][NDIM];

double  p[NMAX];

double  m[NMAX];

main()

{

int ni, nj, i, k;

double eps2;

ni = nj = 2;

for(i=0;i<ni;i++){

for(k=0;k<NDIM;k++){

x[i][k] = 0.0;

}

m[i] = 1.0;

}

x[0][0] = 1;

x[1][0] = -1;

eps2 = 0.01;

h3open_();

accel_by_harp3_separate_noopen_(&ni, x, &nj,x,m,a,p,&eps2);

for(i=0;i<ni;i++){

printf("X %17.8g %17.8g %17.8g M %17.8g\n",

x[i][0], x[i][1], x[i][2], m[i]);

printf("A %17.8g %17.8g %17.8g P %17.8g\n",

a[i][0], a[i][1], a[i][2], p[i]);

}

h3close_();

}

sample makefile:

minimal.h3: minimal.f

f77 -o minimal.h3 -u minimal.f  -L/usr2/makino/src/harp3board -lharp3  -lrt -lm

minimal.g4a: minimal.f

f77 -o minimal.g4a -u minimal.f  -L/usr2/makino/src/pci-harp3/export/library -lg4a -lrt -lm

cminimal.h3: minimal.c

cc -o cminimal.h3 minimal.c  -L/usr2/makino/src/harp3board -lharp3  -lrt -lm

H3: for old interface (can run on oppenheimer and pineau)

G4A: for new PCI interface (can run on steinberger, latour, muscat)

sample output:

<pineau:/usr2/makino/src/harp3board>make -f Makefile.minimal ; minimal.h3

f77 -o minimal.h3 -u minimal.f  -L/usr2/makino/src/harp3board -lharp3  -lrt -lm

(set_number_of_boards_from_environment) : nb=1

(set_firstcluster_from_environment) : nb=-1

(setup_offsets) nchips = 62

X     1.0000000       0.00000000E+00   0.00000000E+00 M     1.0000000

A   -0.24906534       0.00000000E+00   0.00000000E+00 P    -10.499375

X    -1.0000000       0.00000000E+00   0.00000000E+00 M     1.0000000

A    0.24906534       0.00000000E+00   0.00000000E+00 P    -10.499375

<pineau:/usr2/makino/src/harp3board>make -f Makefile.minimal cminimal.h3

cc -o cminimal.h3 minimal.c  -L/usr2/makino/src/harp3board -lharp3  -lrt -lm

<pineau:/usr2/makino/src/harp3board>cminimal.h3

(set_number_of_boards_from_environment) : nb=1

(set_firstcluster_from_environment) : nb=-1

(setup_offsets) nchips = 62

X                 1                 0                 0 M                 1

A       -0.24906534                 0                 0 P        -10.499375

X                -1                 0                 0 M                 1

A        0.24906534                 0                 0 P        -10.499375

<pineau:/usr2/makino/src/harp3board>